Experiment 06

VLAN (Virtual Local Area Network) and VLAN Trunking

Name: Lakshdeep     UID: 24BCF10059

# Aim

To understand what Virtual LANs (VLANs) are and how VLAN trunking works — essentially, how a single physical switch can be carved up into several independent broadcast domains — and to actually configure and test this behaviour, along with IEEE 802.1Q trunking between two switches, in Cisco Packet Tracer.

# Objectives

- Get a working understanding of how a Layer 2 switch forwards frames using its MAC address table.

- Be able to tell a Layer 2 switch apart from a Layer 3 switch.

- Understand what a VLAN is and why it's used to keep broadcast domains separate.

- Know the difference between an access port and a trunk port.

- Understand how VLAN tagging works under the IEEE 802.1Q standard.

- Set up VLANs on a single switch and confirm that devices in different VLANs can't reach each other.

- Build a trunk link between two switches and confirm that one VLAN can legitimately span across both.

# Software / Tools Required

- Cisco Packet Tracer

- Windows Operating System

# Theory

A) Layer 2 Switch and the MAC Address Table

A Layer 2 switch works at the Data Link layer of the OSI model, and it forwards Ethernet frames purely on the basis of MAC addresses. Say PC1 wants to send something to PC2 — it puts its own MAC address as the source and PC2's MAC address as the destination inside the frame. The switch looks at that destination address, checks it against a table it keeps internally (the MAC Address Table), and sends the frame out only through the port where that address was originally learned. This lookup-and-forward process is basically what every Layer 2 switch does, including the Cisco 2960 series switches used in this experiment.

Fig. 6.1 — A Layer 2 switch forwards a frame only to the port associated with the destination MAC address

B) Layer 2 Switch vs Layer 3 Switch

A Layer 3 switch is essentially a Layer 2 switch with IP-based routing built in. While an ordinary Layer 2 switch can only move frames around based on MAC addresses, a Layer 3 switch can also route packets between different IP subnets. That's what makes inter-VLAN routing possible without needing a separate external router.

| Feature | Layer 2 Switch | Layer 3 Switch |
| --- | --- | --- |
| Primary OSI Layer | Layer 2 | Layer 2 + Layer 3 |
| Uses MAC address | Yes | Yes |
| Uses IP address for routing | No | Yes |
| Switching within a VLAN | Yes | Yes |
| Routing between VLANs | No | Yes (Inter-VLAN Routing) |
| Typical example | Cisco 2960 | Cisco 3560 |

C) What is a VLAN?

VLAN stands for Virtual Local Area Network. Normally, without VLANs, every single device plugged into a physical switch ends up in the same broadcast domain. A VLAN lets you logically split one physical switch into multiple independent broadcast domains — for instance, one for HR and a separate one for Sales — without buying a separate switch for every department. Devices sitting in different VLANs simply can't talk to each other unless a router or Layer 3 switch is set up to route traffic between them.

Fig. 6.2 — Left: one physical switch logically divided into VLAN 10 and VLAN 20. Right: two switches, each with members of both VLANs, joined by a trunk link

D) Why Do VLANs Separate Devices?

By default, a switch floods broadcast traffic out to every port in a broadcast domain. Once VLANs are configured, though, the switch keeps its ports logically separated, so a broadcast generated inside VLAN 10 never reaches any port in VLAN 20, and vice versa — even if both VLANs happen to live on the exact same physical switch.

E) Limitations of a Traditional (Non-VLAN) LAN and How VLANs Address Them

| Limitation of a Flat LAN | How VLANs Help |
| --- | --- |
| Every device shares one large broadcast domain. | A physical LAN gets split into several smaller logical broadcast domains, cutting down on unnecessary broadcast traffic. |
| Departments are hard to keep separate. | Each department (Accounts, HR, IT, etc.) can simply be placed in its own VLAN. |
| Weak isolation between users. | Devices in different VLANs can't exchange traffic directly unless routing is explicitly set up. |
| Broadcasts reach devices that don't need them. | Broadcasts stay confined within their own VLAN, which improves overall network performance. |
| Network grouping is tied to physical location. | VLAN membership is logical, so two PCs sitting in different rooms or floors can still belong to the same VLAN. |
| Relocating a user often means re-cabling. | Moving a user to a different group is just a matter of reassigning their switch port to another VLAN. |
| A large LAN becomes hard to manage. | Breaking it into smaller VLANs keeps things organised and much easier to troubleshoot. |
| Extra switches are needed for every extra department. | Several logical networks can share the same physical switch hardware, which saves cost. |

F) Access Ports and Trunk Ports

An access port belongs to exactly one VLAN and is typically used to connect end devices like PCs, printers, or servers — the traffic passing through it is untagged. But when two switches both need to carry the same set of VLANs between them, a single access-style link simply can't handle more than one VLAN at a time, so a trunk port is used instead. A trunk port can carry traffic for multiple VLANs over one physical link — which is exactly how Switch1 and Switch2 are connected in Part B of this experiment.

| Access Port | Trunk Port |
| --- | --- |
| Normally carries a single VLAN. | Carries multiple VLANs over one link. |
| Usually connects an end device. | Usually connects two network devices (switch-to-switch or switch-to-router). |
| Example: PC → Switch. | Example: Switch → Switch. |
| Traffic is untagged toward the end device. | VLAN membership is identified using a VLAN tag. |
| e.g. Fa0/1 assigned to VLAN 10. | e.g. Gi0/1 carrying VLAN 10 and VLAN 20 together. |

G) VLAN Tagging — IEEE 802.1Q

When a frame goes across a trunk link, the switch on the other end has no way of knowing which VLAN that frame actually belongs to — unless that information is carried along with the frame itself. VLAN tagging solves this: as a frame heads out over the trunk, the sending switch inserts a small tag into the Ethernet frame recording its VLAN ID (say, VLAN ID = 10). The receiving switch reads that tag, figures out the frame belongs to VLAN 10, strips the tag off if it's heading to an access port, and forwards it only to ports belonging to VLAN 10. IEEE 802.1Q is the standard tagging mechanism used for all of this.

H) VLAN Trunking vs 802.1Q vs VTP — Avoiding Confusion

These three terms often get mixed up. VLAN Trunking is just the general idea of carrying multiple VLANs over a single physical link. IEEE 802.1Q is the actual open standard used to tag frames so VLAN information survives the trip across that trunk. VTP (VLAN Trunking Protocol), despite what its name suggests, does something quite different — it's used to distribute and keep VLAN configuration information in sync across several Cisco switches within the same VTP domain, and it isn't itself responsible for carrying multi-VLAN traffic over a link.

| Trunking Standard | Type | Status / Use |
| --- | --- | --- |
| IEEE 802.1Q | Open industry standard | Current, most widely used VLAN trunking / tagging method |
| ISL (Inter-Switch Link) | Cisco proprietary | Legacy, largely obsolete on modern equipment |

IEEE 802.1Q trunking was the method used throughout this experiment.

# Procedure

Part A: VLAN Segmentation on a Single Switch

A single switch was used to host two departments — HR and Sales — as two separate VLANs, to show that devices sitting in different VLANs can't reach each other even though they're physically connected to the same switch.

| Device | VLAN | Switch Port | IP Address |
| --- | --- | --- | --- |
| PC0 | 10 (hr) | Fa0/1 | 192.168.1.1 |
| PC1 | 10 (hr) | Fa0/2 | 192.168.1.2 |
| PC2 | 20 (sale) | Fa0/3 | 192.168.2.1 |
| PC3 | 20 (sale) | Fa0/4 | 192.168.2.2 |

- Placed one 2960-24TT switch (Switch0) and four PCs in the Packet Tracer workspace, and connected PC0–PC3 to the switch using copper straight-through cables as per the table above.

- Assigned the IP addresses shown above to PC0 and PC1 (HR / 192.168.1.0 /24) and to PC2 and PC3 (Sales / 192.168.2.0 /24).

- Opened Switch0's CLI and moved into privileged and global configuration mode using enable, then configure terminal.

- Created the two VLANs — vlan 10 followed by name hr, and vlan 20 followed by name sale.

- Assigned the access ports to their VLANs: on fa0/1 and fa0/2, ran switchport mode access and switchport access vlan 10; on fa0/3 and fa0/4, ran switchport mode access and switchport access vlan 20.

- Verified the setup with show vlan brief, then tested connectivity — PC0 ↔ PC1 and PC2 ↔ PC3 worked fine (same VLAN), while PC0 ↔ PC2 failed as expected (different VLANs, no routing configured).

Part B: VLAN Trunking Between Two Switches

Since a real organisation rarely fits neatly on a single switch, the same two VLANs — hr and sale — were then spread across two separate switches, with one member of each department connected to each switch, and the two switches joined by a single 802.1Q trunk link. This was meant to demonstrate that a trunk allows a VLAN to legitimately span multiple physical switches.

| Switch | Device | VLAN | Access Port | IP Address |
| --- | --- | --- | --- | --- |
| Switch1 | PC4 | 10 (hr) | Fa0/1 | 192.168.1.1 |
| Switch1 | PC5 | 20 (sale) | Fa0/3 | 192.168.2.1 |
| Switch2 | PC7 | 10 (hr) | Fa0/1 | 192.168.1.2 |
| Switch2 | PC6 | 20 (sale) | Fa0/3 | 192.168.2.2 |

- Placed two 2960-24TT switches (Switch1, Switch2) and four PCs. Connected PC4 and PC5 to Switch1, and PC7 and PC6 to Switch2, using straight-through cables, and then connected Switch1's Gi0/1 to Switch2's Gi0/1 to form the inter-switch link.

- Assigned IP addresses to all four PCs as shown above — note that hr keeps the 192.168.1.0 /24 range and sale keeps 192.168.2.0 /24, regardless of which physical switch the PC happens to be attached to.

- On Switch1, created the VLANs and assigned the access ports exactly as in Part A: vlan 10 / name hr, vlan 20 / name sale, then set interface fa0/1 to access mode on vlan 10, and interface fa0/3 to access mode on vlan 20.

- Configured the inter-switch link as a trunk using interface gigabitEthernet0/1, followed by switchport mode trunk. This is really the single most important step in the whole experiment — without it, the link could only carry one VLAN's traffic, or none at all.

- Repeated the identical VLAN and trunk configuration on Switch2.

- Checked the VLAN table before and after the trunk was configured using show vlan, confirmed the trunk was active with show interfaces trunk, and inspected the learned addresses using show mac address-table.

- Tested connectivity: PC4 ↔ PC7 (both hr, on different switches) and PC5 ↔ PC6 (both sale, on different switches) succeeded because the trunk carries both VLANs; PC4 ↔ PC5 (different VLANs) still failed, since no inter-VLAN routing had been set up.

Reference Configuration — Switch1

enable configure terminal vlan 10 name hr exit vlan 20 name sale exit interface fa0/1 switchport mode access switchport access vlan 10 exit interface fa0/3 switchport mode access switchport access vlan 20 exit interface gigabitEthernet0/1 switchport mode trunk exit

The same configuration (with fa0/1 carrying PC7 and fa0/3 carrying PC6) was repeated on Switch2.

# Output

The completed topologies for Part A (single-switch VLAN segmentation) and Part B (two switches joined by a trunk) are shown below, as built and tested in Cisco Packet Tracer.

Fig. 6.3 — Left: HR and Sales as two VLANs on a single switch (Switch0). Right: HR and Sales spread across Switch1 and Switch2, joined by a Gi0/1–Gi0/1 trunk

The CLI output below, captured on one of the switches, shows the VLAN table before and after the trunk was configured on Gi0/1. VLAN 10 shows up named hr (port Fa0/1) and VLAN 20 as sale (port Fa0/3), exactly as configured — and once Gi0/1 is converted into a trunk, it disappears from VLAN 1's list of access ports, since a trunk port doesn't belong to just one VLAN anymore.

Fig. 6.4 — show vlan output before and after configuring Gi0/1 as an 802.1Q trunk (switchport mode trunk)

# Observation

- In Part A, PC0 could reach PC1 and PC2 could reach PC3 (same VLAN), but PC0 could not reach PC2 (different VLANs on the same switch).

- In Part B, PC4 could reach PC7 and PC5 could reach PC6, even though each pair sits on a different physical switch — this worked because the Gi0/1 trunk carried both VLAN 10 and VLAN 20, tagged with 802.1Q.

- PC4 still could not reach PC5, which confirms that a trunk carries multiple VLANs side by side without merging them — devices in different VLANs stay isolated unless inter-VLAN routing is added.

- Once Gi0/1 was converted to a trunk, it dropped out of VLAN 1's own port list in the show vlan output, confirming that a trunk port is no longer tied to a single access VLAN.

# Result

VLAN segmentation and VLAN trunking were successfully studied and implemented in Cisco Packet Tracer. Two VLANs (hr and sale) were created and verified to be isolated from one another on a single switch, and were then successfully extended across two switches using an IEEE 802.1Q trunk link on Gi0/1 — allowing devices in the same VLAN to communicate with each other regardless of which physical switch they were plugged into.

# Learning Outcomes

- Understood how a Layer 2 switch forwards frames using its MAC address table, and how that differs from the routing behaviour of a Layer 3 switch.

- Understood the concept of a VLAN as a logical broadcast domain that's independent of physical switch boundaries.

- Learned to create VLANs and assign access ports to them using the switchport access vlan command.

- Understood the difference between an access port and a trunk port, and why a trunk becomes necessary once a VLAN spans more than one switch.

- Learned how IEEE 802.1Q tagging lets a receiving switch identify which VLAN a frame on a trunk actually belongs to.

- Understood the distinction between VLAN trunking (the concept), 802.1Q (the tagging standard), and VTP (a configuration-distribution protocol) — three related but genuinely different ideas.

- Verified VLAN and trunk configuration practically using show vlan, show interfaces trunk, and show mac address-table, and confirmed both inter-VLAN isolation and intra-VLAN connectivity using ping.